## 🔧 HOW TO BLOCK VIDEOS, MUSIC, STREAMING ON ZTE (MTN) ROUTER

### ✅ STEP 1: Login to the Router Admin Interface

- Connect to your ZTE router via Wi-Fi or LAN.

- Open a browser and go to:

perl

CopyEdit

http://192.168.0.1/  or  http://192.168.1.1/

- Login with:

  - Username: admin

  - Password: admin or check the sticker under the router.

### ✅ STEP 2: Enable URL Filtering (Website Blocking)

- Go to:
Security → URL Filter

- Enable URL Filter or Parental Control.

- Add URLs you want to block:

CopyEdit

youtube.com

m.youtube.com

netflix.com

spotify.com

facebook.com

tiktok.com

soundcloud.com

dailymotion.com

vimeo.com

mp3juices.cc

y2mate.com

- Click Apply / Save.

📝 Some ZTE routers don’t support wildcards (*.youtube.com) — if it doesn't accept it, just input both main and mobile versions of the sites.

### ✅ STEP 3: Block These for Specific Devices (Optional)

- Go to Device Management or Connected Devices

- Copy the MAC address or IP of the specific device

- Return to Security → Access Control or Parental Control

- Add the MAC/IP and assign the filter to it

### ✅ STEP 4: Change DNS to OpenDNS or CleanBrowsing

This helps block video/music sites using external DNS filtering.

#### How to change DNS:

- Go to Network Settings → LAN Settings or DHCP Settings

- Find DNS fields:

  - Primary DNS: 208.67.222.123

  - Secondary DNS: 208.67.220.123 (OpenDNS Family Shield)

✅ You can also use:

- CleanBrowsing Family DNS:

  - Primary: 185.228.168.168

  - Secondary: 185.228.169.168

- Click Apply / Save

- Restart the router

### ✅ STEP 5: Block VPN & Streaming Ports (Optional)

VPNs can bypass filters. Some ZTE routers allow you to block certain ports:

- Go to Security → Firewall Settings

- Block:

  - TCP/UDP Ports: 1194, 1701, 500, 4500, 443 (optional)

Be careful: blocking port 443 will affect most secure websites.

### ✅ STEP 6: Monitor & Limit Bandwidth (if available)

- Go to Traffic Statistics or Data Usage

- See devices using high data

- Some models allow you to limit speed or disconnect devices

## 🚫 Limitations of ZTE MTN Routers:

- Most MTN-branded ZTE routers have limited parental control features.

- If URL filtering or DNS doesn’t give you full control, consider using:

  - A better router with advanced filtering (TP-Link Archer, ASUS, etc.)

  - Or installing a filtering app like SafeDNS, OpenDNS, or CleanBrowsing on all devices.

Would you like me to generate a downloadable blocklist or help you configure OpenDNS with category filtering (e.g., block all video/music/social media at once)?